- Design on-off temperature controller with following specification

  - Span 0 to 400oC

  - Output sensitivity of signal conditioner is 10mv/oC

  - Give potentiometer to set desired temp

  - Assume 3 digit DPM with input range of 0-1volts

  - Hysteresis desired is 5% of span

  - Use relay of 2 CO, 10Amps AC contacts, 12vol, 60Ω as actuator

Solution

The span is 400-0=400DC

The sensor output would be 0 to 4Volts (10mv/DC X 400DC)

Use microchip 4.096V reference voltage source. Use a series combination of resistor and a potentiometer in across the reference voltage source to get 4volts exactly across the potentiometer

The Reference voltage to set the desired set point would be 4 Volts.

The DMP sensitivity would be 1v/1000= 1mv. Apply attenuation of 1/10 to display process temperature

Design difference amplifier with gain

The hysteresis is UTP-LTP=5% of span i.e. 4volts = 0.05 X 4Volts= 0.2Volts

Assuming power supply of the Opamp is +-12Volts the saturation voltage of the opamp would be +-10Volts

We shall keep UTP=0.1Volt and LTP=-0.1 volt

UTP=0.1=β X Vosat= β X 10V, Hence β=0.01 = R2/(R1+R2)

Select R1=100K, then R1 would be 1.010Kohms= ~1KOhms

Use transistor BC337 to drive the relay of 12V, 60 Ohms

The required IC to drive relay is VCC-VCE/60ohms=(12-0.1)/60ohms=198ma =~ 200ma, The required IB to drive BC337-40 would be

200ma/ β= 200ma/ 250 = ~1ma Hence Rb1= (Vsat-Vbe)/Ib= 10- 0.7=9.3/1ma= 9.3KOhms= ~10K

Compute Rb2

The reverse leakage current of B337-40 is 100na @room temperature

Assume room temperature is 25DC and maximum operating temperature of the controller is 85DC then the range would be 85-25=60 or 6 decades

Then the ICBO @85DC would be 2^6 X 100na= 6400na=6.4uA

The 6.4uA X Rb2 <=0.1Volts Hence Rb2 should be =< 15.6Kohms =~15Kohms

Put a diode 1N4007 across Rb2 to protect the base emitter junction of BC337 from negative saturation voltage of the Schmitt trigger.

Put a diode 1N4007 across relay coil to protect BC337 voltage reversal spikes of relay coil.